# PURPOSE

INCLUSION OF AIRCRAFT ON AOC

    - This Advisory Circular (AC) provides guidance to air operators seeking to add a variant of an aircraft type currently approved under their Air Operator Certificate (AOC) for commercial air transport operations carrying passengers and cargo

    - This Advisory Circular (AC) also provides guidance to air operators seeking to add a new type of aircraft to their Air Operator Certificate (AOC) for commercial air transport operations carrying passengers and cargo.

    - This Advisory Circular (AC) is an Initial issue dated February 2020.

# REFERENCES

The Civil Aviation (Air Operators Certification and Administration) Regulations as amended.

# GENERAL

    - The addition of a variant aircraft to an existing fleet of that aircraft type requires TCAA approval before any operations of that aircraft. TCAA must—

- Determine that this aircraft is compatible with the operations and maintenance procedures previously approved for the specific aircraft fleet; and

- Ensure that any differences between the variant aircraft and the specific fleet have been addressed by the air operator.

    - This AC outlines the preparation and submission of a formal application for the addition of the aircraft. It also outlines the overall process that will be followed by TCAA inspector during the document conformance evaluation and subsequent inspections and demonstrations necessary to this addition.

    - The air operator seeking the addition of a variant aircraft will be subject to the evaluations and inspections listed in this AC.

    - The addition of a new aircraft type to an air operator certificate requires TCAA approval before any operations of that aircraft-specific type by the air operator.

    - This AC outlines the preparation and submission of a formal application for the addition of the new aircraft-specific type to an existing air operator certificate. It also outlines the overall process that will be followed by TCAA inspector during the document conformance evaluation and subsequent inspections and demonstrations necessary to the addition of the new aircraft type to the AOC.

    - The air operator seeking the addition of a new aircraft type to an existing air operator certificate will be subject to the evaluations and inspections listed in this AC.

  - APPLICATION AND PROCESS: ADDING AIRCRAFT (SAME MAKE/MODEL) TO AOC

    - DETERMINE THE COMPLEXITY OF APPROVAL PROCESS

      - How Variant is the Variant Aircraft?

      - TCAA highly recommends that the AOC holder conduct an evaluation of the complexity of the differences between the proposed aircraft and the existing fleet of that aircraft type. This evaluation should be accomplished using the “Evaluate variant Aircraft (Same Make/ Model” checklist located in Appendix A to this AC.

      - The AOC holder’s evaluation team should consist of aircraft type-specific qualified operations and airworthiness personnel.

      - This completed evaluation form will be the basis for TCAA decision regarding the extent of the formal application and approval process.

# DETERMINE THE COMPLEXITY OF THE DIFFERENCES

      - The AOC holder’s evaluation team will determine, for each element of the evaluation checklist, the complexity level assigned to that element from the following listing—

        - Level 0 – No document revision or personnel briefing or training required.

        - Level 1 – Document revision required, but no personnel briefing or training required.

        - Level 2 – Document revision required and personnel briefings required to ensure workforce understand differences.

        - Level 3 – Document revision required and formal ground training required to ensure workforce understand complexity of differences.

        - Level 4 – Document revision required, formal ground training required and flight training required to ensure pilots are aware of specific flight characteristics.

      - The completed evaluation checklist with a complexity level assigned to each element must be formally provided to TCAA.

Note: 1. If all of the complexity levels are determined to be 1 or 2 only, the application process will be limited to the evaluation of the submitted documents and observation of briefings.

2. If any of the complexity levels are determined to be 3 or 4, the formal application process is required. TCAA will determine the extent of the process by applying the submission requirements of this AC.

# TCAA PRELIMINARY EVALUATION

      - TCAA will assign a team of type-qualified operations and airworthiness inspectors to go to the aircraft to audit the AOC holder’s evaluation.

      - This team will review each element to confirm the assigned complexity levels before a TCAA decision is made regarding the extent and complexity of the application.

      - TCAA will then issue a formal letter of its decision regarding the formal application document submissions and the inspection and demonstrations that will be required.

# IF FORMAL APPLICATION IS REQUIRED

If formal application is required, use the guidance in item 5.0 (Adding New Aircraft type to AOC) below for the processing of the applications and the formal inspection and demonstrations required.

# PROCESS & APPLICATION: ADDING NEW AIRCRAFT TYPE TO AOC

    - Using the Tables in This Circular,

# ABBREVIATIONS FOR TYPE OF OPERATOR

The following abbreviations are used throughout this document to indicate the requirements for each group of applicants—

- LC = large aircraft, no passenger seats

- L = large aircraft, 20+ passenger seats

- C = aircraft with 10 to 19 passenger seats

- B = Charter Operations with 9 or less passenger seats

- S = Charter Operations with 9 or less passenger seats operated by a single pilot

- H = helicopter operator

# APPLICABLE REQUIREMENTS

- Throughout this advisory circular, different tables will be used to outline the requirements applicable to the general groupings of applicants.

- The checkmark [ ] in a column will indicate that the requirement listed on that row is applicable to that particular group. It is very important to the understanding of the requirements outlined in this advisory circular that the reader should not go beyond this point without the concept of how the checkmark symbol is used in these tables. The darkened cells indicate there is not a requirement for that document or manual. That checkmark indicates that the particular manual, document or record is required for the general group of applicants.

- In the example below, “Document 1” would be required only for groupings LC and L (not for C, B, S, or H). “Manual 2” would be required only for groupings B, S and H (not for LC, L or C.

| EXAMPLE OF APPLICABLE REQUIREMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Copies | Description | LC | L | C | B | S | H |
| 1 | Document 1 |  |  |  |  |  |  |
| 2 | Manual 2 |  |  |  |  |  |  |

# PRE-APPLICATION

An applicant for inclusion of a new type of Aircraft onto an AOC will need an initial, personal briefing from the TCAA team that will be making these evaluations. This is through a step called “Pre- Application Meeting.”

# PRE-APPLICATION STATEMENT OF INTENT

- The AOC holder should complete a “Pre-Application Statement of Intent. The form provides TCAA advance notice of the applicant’s intentions.

- The applicant should send this form to TCAA at least 60 working days before they would like to begin commercial air transport flights using the new aircraft type. (If the applicant needs help understanding the form, they should schedule an informal meeting with TCAA for assistance in its completion.)

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Pre-Application Statement of Intent (PASI) |  |  |  |  |  |  |

- This form is simply a general statement of the applicant’s intentions. It does not commit the applicant to hire any persons or buy any aircraft. The purpose of the form is to state his intentions to add a new or variant aircraft to an existing aircraft type-specific fleet.

# PRE-APPLICATION MEETING

      - After TCAA has reviewed the applicant’s pre-application statement, they contact him to schedule a meeting. This meeting is to personally brief the applicant and any other persons of his choice on the certification process and requirements.

      - TCAA will advise the applicant on how to make a formal application, including which documents, contracts and proposed records that must be provided. TCAA will discuss the timelines that will be associated with the approval of a new type of aircraft for the AOC holder.

# CERTIFICATION PROCESS

    - The briefing will outline that the safety certification is a 5-phase process. The Pre- Application Phase is the period between the filing of the statement of intent and the filing of the formal application.

    - During that phase the applicant will be assigned a Certification Project Manager to follow this process and provide answers to the applicant’s questions while he is getting the formal package together.

Note: Throughout the process, it is the applicant’s responsibility to respond to TCAA’s findings and comments to provide the required documents and make the necessary revisions to their processes and documentation.

    - The Formal APPLICATION Phase is the period of time between the submission of the formal application package and actual evaluation of the documents. A TCAA team will meet to determine if the applicant’s application package is complete. The Applicant shall also submit an acceptable Schedule of Events that will be reviewed by TCAA and the applicant during the Formal Application meeting.

- If it’s not complete, the entire package is returned to the applicant for additional work or documents.

- If the applicant’s formal application package is acceptable for further review, the TCAA team will schedule a Formal Application meeting with him, after which phase two is closed.

The next phase is the Document EVALUATION Phase. In this phase TCAA will be reviewing in- depth the applicant’s manuals, documents and proposed records for acceptability. The applicant will probably have to act on a priority basis to revise these documents so that they are acceptable to TCAA. When that phase is complete, next is the Demonstration Phase. During this period, TCAA will inspect the applicant’s facilities, technical support, aircraft, equipment, communications, observe training and checking of crews, and evaluate the AOC holder’s demonstration of capability for operating the new aircraft type.

    - The Final Certification Phase is the one in which TCAA completes their paperwork and issuance of approvals, including the issuance of an Operations Specifications authorizing the use of the new aircraft type in commercial air transport.

# REQUIRED MANUALS, DOCUMENTS & RECORDS

    - The tables below summarize the numbers of each manual, document and records that must be submitted with the formal application. They are separated, first, according to three categories of use—

- General Use. Those manuals, documents and records that provide the general policies and procedures the applicant intends their personnel to use during maintenance and flight operations.

- Fleet Type Specific. Those manuals, documents and records that provide the policies and procedures the applicant intends their personnel to use during maintenance and operations of an aircraft type.

- Individual Aircraft. Those manuals, documents and records that must be available for the individual aircraft by serial number.

For clarity, the tables are further separated according to whether the submission requirement is for a—

  - Manual;

  - Document; or

  - Record

# MODIFICATIONS TO GENERAL USE MANUALS, DOCUMENTS & RECORDS

The applicant has already provided the manuals, documents and records relating to the overall operations and/or maintenance necessary to an AOC holder. Only the revisions to those manuals that are necessary to include the new aircraft type should be submitted. Hence no submission is required for manuals not revised.

# MODIFIED GENERAL USE MANUALS

      - If any of the following manuals relating to overall policies and/or procedures for the operations and maintenance of an AOC were modified to accommodate the new aircraft type, those modified pages must be provided as a revision with the application package—

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Operations Manuals (GOM Parts A, B, C and AFM) |  |  |  |  |  |  |
| 2 | Cabin Crew Member Manual (CCM) |  |  |  |  |  |  |
| 2 | Training Manual (GOM Part D) |  |  |  |  |  |  |
| 2 | Maintenance Control Manual (MCM) |  |  |  |  |  |  |
| 2 | Line Station Manual |  |  |  |  |  |  |
| 2 | Ground Handling Manuals |  |  |  |  |  |  |

      - If any of the separate program manuals related to specific operational policies and/or procedures were revised, those revisions shall be provided with the applicant’s formal application. (The need for new program manuals will be identified during the Pre-Application Meeting.)—

# MODIFIED GENERAL USE DOCUMENTS

If any of the following documents relating to the overall operations and/or maintenance of an AOC were revised or added to accommodate the new aircraft type, that revision must be submitted with the application—

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Resumes for persons nominated for training/ checking |  |  |  |  |  |  |
| 1 | Ground Handling Arrangements |  |  |  |  |  |  |
| 1 | Contracts for Training/Facility/Personnel/ Simulators |  |  |  |  |  |  |
| 1 | Other contracts? |  |  |  |  |  |  |

# MODIFIED GENERAL USE RECORDS

| If any of the following records relating to the overall operations and/or maintenance of an AOC were revised or added to accommodate the new aircraft type, that revision must be submitted with the application—Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Aircraft Journey/Tech-log |  |  |  |  |  |  |
| 2 | Maintenance Deferred Defects Log |  |  |  |  |  |  |
| 2 | Maintenance Condition & Summary Record |  |  |  |  |  |  |
| 2 | Operations Flight Planning Record |  |  |  |  |  |  |
| 2 | Flight Crew Qualification Record |  |  |  |  |  |  |
| 2 | Cabin Crew Qualification Record |  |  |  |  |  |  |
| 2 | Crew Flight/Duty Record |  |  |  |  |  |  |
|  | Copy of Service Provider Personnel Qualification Records (if required) |  |  |  |  |  |  |

# FLEET TYPE SPECIFIC MANUALS, DOCUMENTS AND RECORDS

The applicant is required to provide manuals, documents and records relating to aircraft-specific fleet operations and/or maintenance that are necessary to an AOC holder.

# FLEET TYPE-SPECIFIC MANUALS

        - The following manuals relating to policies and/or procedures for the operations and/or maintenance of the new aircraft type, the AOC the applicant must submit those manuals with this application—

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Standard Operating Procedures (SOPs) [Checklists/Profiles/Briefings/Limitations] |  |  |  |  |  |  |
| 2 | Pilot Operating Handbook (POH) |  |  |  |  |  |  |
| 1 | Approved Flight Manual (AFM) for each fleet model |  |  |  |  |  |  |
| 2 | Minimum Equipment List (MEL) |  |  |  |  |  |  |
| 1 | Master Minimum Equipment List (MMEL) |  |  |  |  |  |  |
| 1 | Configuration Deviation List (CDL) |  |  |  |  |  |  |
| 2 | Runway Analysis |  |  |  |  |  |  |
| 2 | Aircraft Performance Manual |  |  |  |  |  |  |
| 2 | Fueling/Loading/Servicing Manual(s) |  |  |  |  |  |  |
| 2 | Aircraft Deicing Procedures/Manual |  |  |  |  |  |  |
| 2 | Aircraft Maintenance Program |  |  |  |  |  |  |
| 2 | Supplemental Structures Inspection Program |  |  |  |  |  |  |
| 2 | Corrosion Prevention and Control Program |  |  |  |  |  |  |
| 2 | Aging Aircraft Program |  |  |  |  |  |  |
| 1 | Manufacturer’s Maintenance Inspection Documents |  |  |  |  |  |  |
|  | Aircraft Maintenance Manual |  |  |  |  |  |  |

        - The following separate fleet-specific program manuals related to specific operational policies and/or procedures may be required in the applicant’s formal application—.

Note: The applicable requirements will be discussed during the Pre-Application Meeting.

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Mass & Balance Program Manual |  |  |  |  |  |  |

# FLEET TYPE-SPECIFIC DOCUMENTS

The following documents relating to the operations and/or maintenance of the new aircraft type must be submitted with the application—

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Letter from manufacturer advising the latest revision to their pilot operating manuals |  |  |  |  |  |  |
| 2 | Operating Checklists |  |  |  |  |  |  |
| 2 | Passenger Briefing Cards |  |  |  |  |  |  |
|  | Maintenance Task Cards |  |  |  |  |  |  |
|  | Contract from each Approved Maintenance Organization |  |  |  |  |  |  |
|  | Copy of Certificates & Authorizations from each Maintenance Organization |  |  |  |  |  |  |

# FLEET TYPE-SPECIFIC RECORDS

The following records required for the operations and/or maintenance of the new aircraft type must be submitted with the application—

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Load & Performance Planning Record |  |  |  |  |  |  |

# INDIVIDUAL AIRCRAFT MANUALS, DOCUMENTS & RECORDS

The applicant is also required to provide manuals, documents and records relating to individual aircraft during the certification.

# INDIVIDUAL AIRCRAFT MANUALS

The following manuals should be provided by the applicant in the formal application package: -

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Approved Flight Manual |  |  |  |  |  |  |

=

Must be in aircraft for all operations of aircraft.

# INDIVIDUAL AIRCRAFT DOCUMENTS

The following documents relating to an individual aircraft should be provided by the applicant in his formal application package: -

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Copy of Certificate of Airworthiness |  |  |  |  |  |  |
| 1 | Copy of Certificate of Registration |  |  |  |  |  |  |
| 1 | Lease of Aircraft (or ownership papers) |  |  |  |  |  |  |
| 1 | Aircraft Insurance Policy |  |  |  |  |  |  |
| 1 | Weight and Balance Report |  |  |  |  |  |  |
| 1 | Installed Equipment List |  |  |  |  |  |  |
| 1 | LOPA |  |  |  |  |  |  |
| 1 | Configuration Conformance Report |  |  |  |  |  |  |

# INDIVIDUAL AIRCRAFT RECORDS

Access to the aircraft records shall be by the applicant at the start of Phase 4 of the process—

| Copies | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Airframe Maintenance Records |  |  |  |  |  |  |
|  | Power plant Maintenance Records |  |  |  |  |  |  |
|  | Propeller Maintenance Records (if propeller-driven aircraft) |  |  |  |  |  |  |
|  | Component Maintenance Records |  |  |  |  |  |  |
|  | Aircraft Survival and Equipment Records |  |  |  |  |  |  |
|  | Flight Deck Voice and Data Recorder Records |  |  |  |  |  |  |

=

These records must be available for inspection in applicant’s designated facilities.

# ADDITIONAL DOCUMENT REQUIREMENTS

  - FLIGHT SAFETY DOCUMENTS SYSTEM

Applicants operating large aircraft will be required to provide their manuals under the flight safety documents system concept.

# PROVISION OF DIGITAL DOCUMENTS FOR COMMENTING

    - Applicants that are required to submit their documents under the flight safety documents system will also provide these documents in digital form on a CD, consolidated into a searchable folder.

    - All other applicants desiring timely and efficient processing of the evaluation of their documents should also provide their documents in digital form as outlined in this Advisory Circular.

  - CERTIFICATION ACTIONS & POLICIES

  - FORMAL APPLICATION REVIEW

    - GENERAL

The applicant’s submission of the formal application and the required documents to TCAA marks the beginning of the Formal Application Review Phase.

# STATUS OF SUBMITTED DOCUMENTS

TCAA will review the application and compile a “Status of Submitted Documents” table. This table will be up-dated throughout the Document Evaluation. As up-dated, a paper copy will be provided to the applicant.

# APPLICATION NOT COMPLETE

      - If the application package is not complete and adequate to begin technical evaluation of the documents, the entire package will be returned to the applicant.

      - It is the applicant’s responsibility to provide a complete and adequate application package.

      - The applicant must take the necessary actions to have a complete and adequate application package before resubmission.

# FORMAL APPLICATION ACCEPTANCE OR REJECTION MEETING

      - If the applicant’s formal application is incomplete and/or in an unacceptable form to allow subsequent technical evaluations, TCAA will have a formal meeting with him to return the entire certification package and discuss the primary unacceptable issues.

      - If the applicant’s formal application is complete and in acceptable form to allow subsequent technical evaluations by the assigned inspectors, TCAA will have a formal meeting with him to address any outstanding scheduling issues before initiating the Document Evaluation Phase.

      - During this meeting, the Schedule Of Events (SOE) submitted by the applicant will be edited by TCAA to include realistic target dates for key events that will occur in the remainder of certification. From this point, TCAA will up-date the SOE to reflect the changing target dates and factors causing the changes. A paper copy of each edited SOE will be provided to the applicant.

# POSSIBLE DELAYS ASSOCIATED WITH A DIFFERENT STATE OF REGISTRY

      - If the State of Registry of the aircraft is not Tanzania, TCAA will notify the appropriate State of our intent to begin a detailed evaluation of the applicant’s documents following the formal application meeting.

      - It will remain the applicant’s responsibility to ensure that the civil aviation authorities of the State of Registry approve the Maintenance Control Manual, Aircraft Maintenance Program, and, in some cases, the Aircraft MEL.

  - Without those approvals, TCAA will not be able to complete the certification process to   allow the use of these aircraft on an AOC.DOCUMENT EVALUATION PHASE CONSIDERATIONS

    - UNSUBMITTED MANUALS, DOCUMENTS OR RECORDS

      - If the applicant fails to submit any document in a timely manner that failure may result in adverse delays in completing the certification of the applicant as an AOC holder.

      - TCAA suggests that the applicant obtain a letter confirming receipt for any document that he submits to TCAA that was not included at the time of the formal application submission.

# DOCUMENT REVISION TURNAROUND

      - The applicant’s failure to act on the requested correction/revision may result in significant  delays to the certification process.

# RULES FOR CORRECTION/REVISION

    - Any approved or accepted document that is returned to the applicant for correction/revision is – after he has taken the corrective actions – a revision to the original document. The applicant’s procedures for revision of manuals and documents must be followed.

    - Do not resubmit it as an “original.” It must be resubmitted as “Revision #?” and so noted  on the document pages revised and in the List of Effective Pages.

# FLIGHT TRAINING CAN BEGIN WHEN….

TCAA has reviewed the applicant’s company and aircraft procedures and checklists and given approval to his Training Program Manual (GOM Part D).

# DEMONSTRATION PHASE

    - This phase consists of a series of sampling inspections by TCAA as the applicant begins training and takes possession of their facilities and aircraft.

    - These inspections may overlap with some document evaluation actions depending on the applicant’s readiness.

# INTERIM (PRE-INSPECTION PHASE) OBSERVATIONS & INSPECTIONS

The following observations and inspections may happen at any point after TCAA has approved the supportive documents for the applicant’s training and maintenance arrangements, even though the Document Evaluation Phase has not yet been completed for all documents requiring TCAA review.

|  | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Operations Training Facilities & Equipment Inspections |  |  |  |  |  |  |
| 2. | Operations Simulator/Training Device Inspections |  |  |  |  |  |  |
| 3. | Operations Training-in-Progress Inspections |  |  |  |  |  |  |
| 4. | Operations Flt Crew Proficiency Check Inspections |  |  |  |  |  |  |
| 5. | Operations Check Pilot Inspections |  |  |  |  |  |  |
| 6. | Cabin Crew Member Competency Check Inspections |  |  |  |  |  |  |
| 7. | Maintenance Training Facility Inspections |  |  |  |  |  |  |
| 8. | Maintenance Training-in-Progress Inspections |  |  |  |  |  |  |

# INSPECTION OF FACILITIES, AIRCRAFT & SUPPORT ARRANGEMENTS

After completion of the Document Evaluation Phase, the following observations and inspections will be conducted to assess that infrastructure and support arrangements are acceptable to the TCAA.

|  | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Operations Structure & Support Inspection |  |  |  |  |  |  |
| 2. | Maintenance Main Base & Facility Inspections |  |  |  |  |  |  |
| 3. | Contractor Maintenance Facility Inspections |  |  |  |  |  |  |
| 4. | Aircraft Records Inspection |  |  |  |  |  |  |
| 5. | Maintenance Aircraft Inspection |  |  |  |  |  |  |

# PREPARATION FOR THE OPERATIONAL INSPECTIONS

The following inspections and demonstrations will be conducted prior to the beginning of the demonstration flights. Note that these inspections and demonstrations must be conducted prior to the demonstration flight days (except in the case of Charter operators with aircraft having 9 or less passenger seats).

|  | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Flight Crew Records Inspection |  |  |  |  |  |  |
| 2. | Cabin Crew Member Records Inspection |  |  |  |  |  |  |
| 3. | Scheduled Services Airport Inspection |  |  |  |  |  |  |
| 4. | Operations Station Inspection |  |  |  |  |  |  |
| 5. | Maintenance Station Inspection |  |  |  |  |  |  |
| 6. | Ground Handling Arrangements |  |  |  |  |  |  |

# DEMONSTRATIONS

    - PRE-OPERATIONS DEMONSTRATIONS

In accordance with TCAA guidance the following pre-operations demonstrations of capability will be conducted.

|  | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Emergency Evacuation Demonstration (20+ Passenger capacity) |  |  |  |  |  |  |
| 2. | Emergency Evacuation Capability Inspection (19 or less Passenger capacity) |  |  |  |  |  |  |
| 3. | Ditching Demonstration (20+ Passenger capacity) |  |  |  |  |  |  |
| 4. | Ditching Capability Inspection (19 or less Passenger capacity) |  |  |  |  |  |  |
| 5 | Procedures (Tabletop) Demonstration |  |  |  |  |  |  |

# DEMONSTRATION FLIGHTS

      - The operational inspections listed below will be conducted during initial certification and will involve the actual flight of the aircraft to allow TCAA to assess the applicant’s ability to conduct flight operations.

      - In the case of large aircraft and commuter operations this will consist of demonstration flights with the other inspections conducted during those operations.

# Demonstration Flights

The following demonstration flights are required—

|  | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Formal Demonstration Flights (For Passenger Authorization) |  |  |  |  |  |  |
| 2. | Formal Demonstration Flights (For EDTO Authorization) |  |  |  |  |  |  |
| 2. | Formal Validation Flights (For Cargo-Only Authorization) |  |  |  |  |  |  |

# Inspections associated with Demonstration Flights

The following inspections should be conducted immediately prior to or during demonstration flights. (Where possible, these inspections should also be conducted prior to or during demonstration flights.)

|  | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Maintenance Control Inspection |  |  |  |  |  |  |
| 2. | Operational Control Inspection |  |  |  |  |  |  |
| 3. | Operations Flight Preparation Records Inspection |  |  |  |  |  |  |
| 4. | Operations Aircraft Exterior Inspection |  |  |  |  |  |  |
| 5. | Operations Cabin Ramp Inspection |  |  |  |  |  |  |
| 6. | Operations Flight Deck Ramp Inspection |  |  |  |  |  |  |
| 7. | Operations Flight Deck Enroute Inspection |  |  |  |  |  |  |
| 8. | Operations PIC Route Check Observation |  |  |  |  |  |  |
| 9. | Operations Cabin Enroute Inspection |  |  |  |  |  |  |
| 10. | Maintenance Cabin Enroute Inspection |  |  |  |  |  |  |

# FINAL CERTIFICATION ACTIONS PHASE CONSIDERATIONS

    - AOC AND OPERATIONS SPECIFICATIONS

      - The following documents are the primary evidence to international civil aviation authorities that an AOC holder has completed the safety certification process and on-going surveillance is being conducted.

      - These must be received by the applicant before conducting any commercial air transport operations. No operations other than those listed in these documents are authorized for an AOC holder or its management, pilots, or aircraft.

|  | Description | LC | L | C | B | S | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air Operator Certificate (AOC) |  |  |  |  |  |  |
|  | Operations Specifications |  |  |  |  |  |  |

____________________________

Director Safety Regulation

APPENDIX A

Evaluate Variant Aircraft (Same Make/Model)

COMPLETE THIS CHECLIST FOR INCLUSION WITH THE FORMAL APPLICATION FOR TCAA APPROVAL TO ADD THIS AIRCRAFT TO THE AOC HOLDER’S FLEET.

| VARIANT AIRCRAFT EVALUATION FOR AIRCRAFT: |  |  |
| --- | --- | --- |
| CHECKLIST DATE: |  | COMPLETED BY: |

        - For each subject area, check NO if there are no differences between this aircraft and the existing AOC holder’s fleet of this make and model.

        - For each subject area, check YES if there is a difference between this aircraft and the existing AOC holder fleet of this make and model.

        - For each element, enter the complexity level of differences (as specified in Section 2 of this AC) in the LEVEL column

        - In the DOC REVISIONS column, enter the manual and chapter or paragraph where revisions were made to accommodate the inclusion of this aircraft in the AOC holder’s fleet of this make and model.

| 1 | FLIGHT DECK | YES | NO | NA | LEVEL | DOC REVISION |
| --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Flight Management System Presentation and Operation |  |  |  |  |  |
| 1.2 | Primary Flight Guidance Presentation and Operation |  |  |  |  |  |
| 1.3 | Other Instrumentation Location and Marking? |  |  |  |  |  |
| 1.4 | Other Switch Location and Operation |  |  |  |  |  |
| 1.5 | Warning Indications and |  |  |  |  |  |
|  | Sounds Presentation? |  |  |  |  |  |
| 1.6 | Circuit Breaker Location |  |  |  |  |  |
| 1.7 | Communications Equipment |  |  |  |  |  |
| 2 | CRITICAL INFORMATION | YES | NO | NA | LEVEL | DOC REVISION |
| 2.1 | Instrument Approach Minimums |  |  |  |  |  |
| 2.2 | Passenger Information Cards |  |  |  |  |  |
| 2.3 | Condensed Checklists |  |  |  |  |  |
| 2.4 | Expanded Checklists |  |  |  |  |  |
| 2.5 | Aircraft Limitations |  |  |  |  |  |
| 2.6 | Aircraft Performance |  |  |  |  |  |
| 2.7 | Aircraft Weight and Balance |  |  |  |  |  |
| 2.8 | Weight and Balance Computations |  |  |  |  |  |

| 2.9 | Operational Flight Plan Computations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2.10 | Aircraft Operation Manual |  |  |  |  |  |
| 2.11 | Minimum Equipment List (Installation & Dispatch) |  |  |  |  |  |
| 2.12 | Training Programme or Syllabi |  |  |  |  |  |
| 2.13 | Operational Bulletin |  |  |  |  |  |
| 2.14 | Maintenance Program |  |  |  |  |  |
| 2.15 | Main Program Bridging Doc |  |  |  |  |  |

| 2.16 | Reliability Program |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2.17 | Engine Condition Monitoring |  |  |  |  |  |
| 2.18 | Illustrated Parts Catalog |  |  |  |  |  |
| 3 | AIRCRAFT SYSTEMS | YES | NO | NA | LEVEL | DOC REVISION |
| 3.1 | Aircraft General |  |  |  |  |  |
| 3.2 | Air Conditioning and Pressurization |  |  |  |  |  |
| 3.4 | Automatic Pilot |  |  |  |  |  |
| 3.5 | APU |  |  |  |  |  |
| 3.6 | Electrical |  |  |  |  |  |
| 3.7 | Emergency Equipment Location and Use |  |  |  |  |  |
| 3.8 | Power plant |  |  |  |  |  |
| 3.9 | Fire Protection |  |  |  |  |  |
| 3.10 | Flight Controls |  |  |  |  |  |
| 3.11 | Fuel |  |  |  |  |  |
| 3.12 | Hydraulics |  |  |  |  |  |
| 3.13 | Ice & Rain Protection |  |  |  |  |  |
| 3.14 | Instrumentation and |  |  |  |  |  |
| 3.15 | Landing Gear |  |  |  |  |  |
| 3.16 | Navigation |  |  |  |  |  |